GRACE ENGINEERING

Plex Connect REST API

Developer Reference & Endpoint Guide

April 2026  |  Author: Shane Waid

# 1. Overview

This document covers the Plex Connect REST API as discovered and tested by Grace Engineering for the purpose of building out CNC cutting tool management infrastructure, workcenter data extraction, and bulk upload automation. It is intended as a working reference for developers integrating with the Grace Plex instance (plexonline.com).

⚠️ This document contains internal API credentials configuration. Do not distribute outside Grace Engineering / Rockwell vendor relationships.

## 1.1  Platform Background

- Grace Engineering runs Plex Classic (plexonline.com), MES+ enabled

- Two product lines: Prime Archery (BOWS) and Montana Rifle Company

- Plex acquired by Rockwell Automation in 2021 — both Classic and UX versions active

- API access provisioned via Plex IAM through the Developer Portal at developers.plex.com

- Rate limit: 200 API calls per minute across all endpoints

## 1.2  What We Are Building

- Bulk Tool Assembly upload — 684 tools from Tool_Database_v2.xlsx into Plex Tool Master

- Tool BOM linkage — tools assigned to routing operations with station positions

- Purchase Order extraction — harvest MSC tool history from PO line items

- Workcenter data — confirm machine codes for tool BOM assignments

- DNC infrastructure — FTP transfer to Brother Speedio 879/880 (192.168.25.79/.80)

# 2. Authentication

All API calls require a subscription key header. There is NO session token or OAuth flow — the key is passed directly on every request.

## 2.1  Getting Credentials

- Go to: https://developers.plex.com/

- Sign in with your Grace company email and IAM credentials

- Navigate to: Account Name → My Apps → Add a New App

- Name the app (e.g. Grace-Engineering-Tools), select API Products, click Create

- Copy the Consumer Key — this is your API key for all requests

- Note: App must be created before any API access works — 401 errors until this step is complete

NOTE: Plex IAM must be activated by your Plex Champion (IT contact) before the developer portal login works. Contact Courtney (IT) to confirm IAM activation before attempting login.

## 2.2  Request Header

Add this header to every API request:

X-Plex-Connect-Api-Key: <your_consumer_key>

CRITICAL: The API key must be in the Headers tab, NOT as a query parameter. Placing it in Params (URL query string) will cause 401 errors even though the key is correct. Confirmed failure mode during testing.

## 2.3  PowerShell Quick Reference

Set environment variables once per session (or permanently):

$env:PLEX_API_KEY = "your_consumer_key_here"

[System.Environment]::SetEnvironmentVariable("PLEX_API_KEY", "your_key", "User")

[System.Environment]::SetEnvironmentVariable("PLEX_BASE_URL", "https://connect.plex.com", "User")

Standard GET request pattern:

Invoke-RestMethod -Uri "$env:PLEX_BASE_URL/collection/v1/resource" -Headers @{"X-Plex-Connect-Api-Key"=$env:PLEX_API_KEY} | ConvertTo-Json -Depth 5

Save to file:

Invoke-RestMethod -Uri "$env:PLEX_BASE_URL/mdm/v1/parts" -Headers @{"X-Plex-Connect-Api-Key"=$env:PLEX_API_KEY} | ConvertTo-Json -Depth 5 | Out-File "C:\Users\shanewaid\Documents\PlexAPI\parts.json"

Search saved JSON files:

Select-String "INDUSTRIAL" "C:\Users\shanewaid\Documents\PlexAPI\suppliers.json"

## 2.4  curl Reference (Command Prompt)

curl -H "X-Plex-Connect-Api-Key: your_key" https://connect.plex.com/mdm/v1/parts

PowerShell 'curl' is an alias for Invoke-WebRequest — NOT real curl. Use 'curl.exe' or use Invoke-RestMethod instead for reliable JSON handling.

## 2.5  Base URLs

| Environment | Base URL |
| --- | --- |
| Production | https://connect.plex.com |
| Test | https://test.connect.plex.com |

# 3. URL Structure

All endpoints follow this pattern:

https://connect.plex.com/{collection}/{version}/{resource}[/{id}]?[filters]

Filter multiple values by repeating the parameter:

https://connect.plex.com/mdm/v1/parts?status=ACTIVE&group=PRIME

Spaces in filter values must be URL-encoded as %20. Example: type=MRO%20SUPPLIES. Unencoded spaces may cause zero results rather than an error.

# 4. Discovered Endpoints

The following endpoints were tested against the Grace production environment. Status reflects actual test results.

## 4.1  Master Data Management (mdm)

| Endpoint | Method | Status / Notes |
| --- | --- | --- |
| mdm/v1/parts | GET | ✅ Confirmed working |
| mdm/v1/suppliers | GET | ✅ Returns data — MSC not found by name (see Section 6) |
| mdm/v1/employees | GET | ❌ 401 — not in API subscription |

## 4.2  Purchasing

| Endpoint | Method | Status / Notes |
| --- | --- | --- |
| purchasing/v1/purchase-orders | GET | ✅ Confirmed working — returns full PO headers |
| purchasing/v1/purchase-order-lines | GET | ⚠️ Endpoint exists — line item detail pending full test |
| purchasing/v1/purchase-orders-lines | GET | ❌ 404 — wrong name (no S on 'order') |
| purchasing/v1/suppliers | GET | ❌ 404 — use mdm/v1/suppliers instead |

## 4.3  Manufacturing

| Endpoint | Method | Status / Notes |
| --- | --- | --- |
| manufacturing/v1/work-centers | GET | ❌ 404 — not in subscription or wrong path |
| manufacturing/v1/operations | GET | ❌ 404 — not in subscription or wrong path |
| manufacturing/v1/routings | GET | ❌ 404 — not in subscription or wrong path |

## 4.4  Inventory & Tooling

| Endpoint | Method | Status / Notes |
| --- | --- | --- |
| inventory/v1/inventory | GET | ❌ 403 — subscription not activated |
| tooling/v1/tools | GET | ❌ 404 — not confirmed |
| tooling/v1/tool-assemblies | GET | ❌ 404 — not confirmed |
| tooling/v1/tool-inventory | GET | ❌ 404 — not confirmed |

ACTION REQUIRED: Contact Courtney (IT) to enable additional API product subscriptions — specifically Manufacturing, Inventory, and Tooling collections. These exist in the platform but are not active on the current app subscription.

# 5. Known Field Structures

The following field structures were confirmed from live API responses against the Grace instance.

## 5.1  Part Record  (mdm/v1/parts)

| Field | Notes / Example |
| --- | --- |
| id | UUID — internal Plex key (e.g. 685994ea-59dd-4819-a287-5761cf676a04) |
| number | Part number string (e.g. D5L.7.31.0.65-GYGY) |
| name | Full part description |
| revision | Part revision letter |
| description | Secondary description field |
| type | Part type (e.g. TAXABLE) |
| group | Product line (e.g. PRIME for Prime Archery) |
| productType | Product category (e.g. BOWS) |
| status | ACTIVE / INACTIVE |
| leadTimeDays | Numeric lead time |
| createdDate | ISO 8601 UTC (e.g. 2025-12-08T15:53:00Z) |

## 5.2  Purchase Order Header  (purchasing/v1/purchase-orders)

| Field | Notes / Example |
| --- | --- |
| id | UUID — use this to pull line items |
| poNumber | PO number string (e.g. 038131) |
| status | New / On Order / Received / Closed |
| type | PO type — MRO SUPPLIES for tool orders |
| supplierId | UUID — does NOT match supplier code (e.g. MSC001) |
| department | Ordering department (e.g. ENGINEERING, Marketing) |
| lineItemCount | Number of line items on this PO |
| lineItemTotal | Total dollar value |
| poDate | PO creation date (ISO UTC) |
| note | Free-text PO note field |
| shipTo | Receiving location code (e.g. POTTER, LAMBS) |

IMPORTANT: The supplierId in API responses is a UUID, NOT the supplier code (MSC001). MSC Industrial is stored as 'MSC INDUSTRIAL SUPPLY' in the display but the UUID must be resolved via the mdm/v1/suppliers endpoint. Initial testing found MSC not matching expected search terms — try 'INDUSTRIAL' as search term in the JSON response.

# 6. Known Issues & Gotchas

## 6.1  Supplier Name Mismatch

MSC is coded as 'MSC001' in the Plex Classic UI but stored under a different name in the MDM. When searching the suppliers JSON dump, try these strings:

- INDUSTRIAL

- MSC INDUSTRIAL

- MSC INDUSTRIAL SUPPLY

- MSC INDUSTRIAL DIRECT

Alternative approach: Filter purchase-orders by type=MRO%20SUPPLIES and extract supplierId UUID directly from a known MSC PO header rather than searching the supplier list.

## 6.2  PO Type Filter Returns Zero Results

Filtering purchase-orders by type string (e.g. ?type=MRO%20SUPPLIES) returned Count: 0 during testing. The type values stored in the API database may differ from the display values shown in the Classic UI.

Recommended fix: Pull all purchase-orders unfiltered, then extract unique type values:

$pos = Get-Content "C:\Users\shanewaid\Documents\PlexAPI\purchase_orders.json" | ConvertFrom-Json

$pos.value | Select-Object -ExpandProperty type | Sort-Object -Unique

## 6.3  HTML Responses Instead of JSON

If an API call returns HTML (DOCTYPE, title 'Postman', body content) instead of JSON, the endpoint URL is wrong and Plex is redirecting to the portal login page. Check for:

- Typos in endpoint path (e.g. purchase-orders-lines vs purchase-order-lines)

- Wrong collection name (e.g. purchasing vs mdm)

- Subscription not enabled for that collection

## 6.4  PowerShell curl Alias

PowerShell's 'curl' command is an alias for Invoke-WebRequest, not GNU curl. This causes different behavior and header handling. Always use:

- Invoke-RestMethod — for JSON APIs (auto-parses response)

- curl.exe — if real curl is installed and needed

- Never use 'curl' bare in PowerShell for REST APIs

## 6.5  Environment Variables Reset on Session End

$env: variables set in PowerShell are session-scoped. To persist across reboots:

[System.Environment]::SetEnvironmentVariable("PLEX_API_KEY", "value", "User")

Then restart PowerShell. The first session after SetEnvironmentVariable must still set $env: manually.

## 6.6  Large Response Size

The mdm/v1/parts endpoint returned 16 MB in initial testing. Implement pagination using limit and offset parameters, or filter aggressively before processing. The rate limit of 200 calls/minute becomes relevant for paginated extraction of large datasets.

# 7. CSV Upload Pipeline (Non-API)

While the REST API is being built out, bulk data can be loaded via Plex's CSV upload system. This does not require API access and is available to engineer-level users today.

## 7.1  Upload Sequence for Tool Management

- Tool Assembly Upload — creates master tool records Step 1

- Tool Inventory Upload — serialized physical instances Step 2

- Tool BOM Upload — assigns tools to part operations Step 3

- Tool Master Process Routing Upload — links to routing operations Step 4

## 7.2  Tool Assembly Upload Template

File: Plex_Tool_Assembly_Upload.csv (generated by tool_db_to_plex.py)

| Column | Required | Notes |
| --- | --- | --- |
| Assembly No | YES | MSC catalog number or internal scheme (GRC-XXXX for custom) |
| Tool Assembly Type | NO | Must match Plex picklist exactly — define picklist BEFORE upload |
| Description | NO | Pulled from Tool_Database_v2 — includes diameter and extra info |
| Part No | NO | Leave blank — assign to routings after upload |
| Part Revision | NO | Leave blank initially |
| Operation | NO | Leave blank initially |
| Tool Assembly Status | YES | Active or Inactive |
| Include in Analysis | NO | 0 or 1 |
| Note | NO | Contains vendor name and MSC URL |
| Location | NO | Set to Assembly No (tool crib location = tool number) |

BEFORE UPLOADING: Define Tool Assembly Type picklist values in Plex first. Types in the upload CSV must match the picklist exactly or rows will fail silently. Test with 5–10 rows before committing all 684 records.

## 7.3  Tool Numbering Convention

| Scenario | Format | Example |
| --- | --- | --- |
| Tool on MSC | MSC catalog number | 00654772 |
| Not on MSC, has mfr# | PREFIX-partnumber | KMT-DNMG432 |
| Custom / special | GRC-XXXX sequential | GRC-0001 |

# 8. Python Script Reference

All scripts live at: C:\Users\shanewaid\Documents\PlexAPI\

## 8.1  plex_api.py

Base API client with authentication, throttling, pagination, and endpoint discovery.

- PlexClient class — handles auth headers and rate limiting automatically

- discover_all() — probes 11 endpoint patterns, saves results to plex_api_discovery.csv

- explore_endpoint() — fetch small sample from any endpoint to inspect field names

- extract_purchase_orders() — dump all POs to CSV

- extract_parts() — dump all parts to CSV

- extract_workcenters() — dump workcenter list to CSV

## 8.2  tool_db_to_plex.py

Converts Tool_Database_v2.xlsx into Plex Tool Assembly upload CSV.

- Input: /mnt/user-data/uploads/Tool_Database_v2__1_.xlsx

- Output: Plex_Tool_Assembly_Upload.csv (684 tools)

- Output: Plex_Tool_Assembly_SKIPPED.csv (3 tools missing Tool_ID)

- Parses messy supplier strings (MSC, GENSWISS, PRE008, SLATER, GARR, etc.)

- Maps 18 tool types to Plex-friendly names

- Sets Location = Assembly No (tool crib location mirrors tool number)

# 9. DNC / Machine Connectivity

Separate from Plex API — program transfer infrastructure for the shop floor.

## 9.1  Brother Speedio 879 & 880 (FTP)

- Machine 879: Target IP 192.168.25.79

- Machine 880: Target IP 192.168.25.80

- Protocol: Native FTP on CNC-D00 control

- Network: 192.168.25.x subnet (confirm IPs with Courtney before assignment)

- Setup path: SYSTEM → Parameter → Network on CNC-D00

- Test: ftp 192.168.25.80 from command prompt after IP assignment

- FTP client: FileZilla (free) or Python ftplib

## 9.2  Citizen L20 / Tsugami / Star (RS-232 via Moxa)

- Hardware: Moxa NPort 5150 (1-port) or NPort 5250 (2-port) — ~$40-60 used

- One Moxa per machine, mounted to cabinet, wired once to RS-232 port

- Moxa converts RS-232 to TCP/IP — machine then accessible over network

- Confirm baud rate, stop bits, and parity from Mitsubishi/Fanuc parameter screen

## 9.3  Haas VMCs (Native Ethernet)

- Sigma 5 machines have native ethernet — no additional hardware needed

- Sigma 1 machines: check for ethernet port on control PCB

- Haas supports SMB network share natively

- Settings → About on control shows software version for capability confirmation

Plex DCS (Document Control System) stores NC programs as master reference copies. Transfer to machines happens separately via FTP/RS-232. Plex is the source of truth; DNC moves the file to the machine at job setup time.

# 10. Open Items & Next Steps

| Task | Owner | Status |
| --- | --- | --- |
| Define Tool Assembly Type picklist in Plex | Shane | ⚠️ Pending |
| Test Tool Assembly CSV upload (5 rows) | Shane | ⚠️ Pending |
| Full 684-tool CSV upload | Shane | ⚠️ Blocked on picklist |
| Enable Manufacturing/Inventory API subscriptions | Courtney (IT) | ⚠️ Pending |
| Resolve MSC supplier UUID via API | Developer | ⚠️ In progress |
| Pull PO line items via purchase-order-lines endpoint | Developer | ⚠️ In progress |
| Confirm PO type filter values from unfiltered dump | Developer | ⚠️ Pending |
| Assign static IPs to Brothers 879 & 880 | Shane / Courtney | ⚠️ Pending |
| Run ethernet cable to Brothers, configure FTP | Shane | ⚠️ Pending |
| Build Python FTP transfer script for Brothers | Shane | ⚠️ Pending |
| Moxa NPort procurement for Citizen/Tsugami machines | Shane | ⚠️ Pending |
| Attach setup sheets to Part Operation level in DCS | Shane | ⚠️ In progress — 1004467 |
| Regen API key (exposed in screenshots) | Shane | ⚠️ Before production use |
| Post to Plex Community forum — tool management | Shane | ⚠️ Draft ready |

Grace Engineering  |  Prime Archery / Montana Rifle Company  |  Internal Use Only